Texas Instruments, Inc.

P98_SST_DMD – HSSI Modelling Documentation

IBIS and s-parameter/HSPICE models

1/29/2020

The P98_SST_DMD is modelled in two parts.  The lower speed digital pins are simulated by means of an IBIS model.  The high speed serial interface (HSSI) pins are modelled by means of package s-parameter models and silicon termination HSPICE models.  The models are listed below.

P98_SST_DMD

P98_SST_DMD.IBS   -  digital pin IBIS model for the P98_SST_DMD

NB321BET99-120170525_bw_macroA.s36p – package s-parameter model for HSSI macro A interface

NB321BET99-120170525_bw_macroB.s36p – package s-parameter model for HSSI macro B interface

NB321BET99-120170525_bw_macroC.s36p – package s-parameter model for HSSI macro C interface

NB321BET99-120170525_bw_macroD.s36p – package s-parameter model for HSSI macro D interface

rx_afe_clocklane_model_nom.hsp – nominal HSSI clock lane receiver HSPICE model

rx_afe_clocklane_model_low.hsp – low termination resistance and fast silicon corner HSSI clock lane receiver HSPICE model

rx_afe_clocklane_model_high.hsp – high termination resistance and slow silicon corner HSSI clock lane receiver HSPICE model

rx_afe_datalane_model_nom.hsp – nominal HSSI data lane receiver HSPICE model

rx_afe_datalane_model_low.hsp – low termination resistance and fast silicon corner HSSI data lane receiver HSPICE model

rx_afe_datalane_model_high.hsp – high termination resistance and slow silicon corner HSSI data lane receiver HSPICE model

The recommended simulation tool is ADS.  Within ADS, the above models can be imported for use in the system simulation.  The HSSI interface should be connected as below.  The boxes on the left would include the driver model plus the interconnect models from the driver to the receiver.  These models will input to the package s-parameter model on the LGA/PGA side.  The package s-parameter model “DIE” side then interfaces with the corresponding HSPICE models, whose port names are shown on the right.

For questions concerning ADS with how to import a netlist or s-parameter, please see the following documentation on Keysights’ website.

https://edadocs.software.keysight.com/ads2020/input-output/data-input-output/netlist-translator-for-spice/importing-a-netlist-file

https://edadocs.software.keysight.com/ads2020/components/components-analog-rf/introduction-to-circuit-components/data-file-components/snp-n-port-s-parameter-file